TECHNICAL DOCUMENTATION

Desktop Computer Troubleshooting Procedure

Issue: Black Screen on Power-On (Task 1)

Group Members

Member 1: Abiy Yrsaw

Member 2: Blen Belete

Member 3: Sofonyas Yared

Member 4: Mahdot F/Sbhat

Member 5: Adugna Emru

Date:  30/06/2026

# Table of Contents

Table of Contents	2

1. Problem Statement	3

2. Symptom Summary	3

3. Possible Causes	3

4. Tools and Materials Required	3

5. Safety Precautions	4

6. Diagnostic and Repair Procedure	4

Step 1 – Check External Connections	4

Step 2 – Observe Power-On Behavior	4

Step 3 – Investigate Power Issues	4

Step 4 – Investigate Video and Memory Issues	5

Step 5 – Check for Overheating or Component Failure	5

Step 6 – Reset BIOS/CMOS	5

Step 7 – Isolate the Faulty Component	5

7. Verification	5

8. Documentation and Reporting	5

9. Conclusion	6

TECHNICAL DOCUMENTATION

Desktop Computer Troubleshooting Procedure

Issue Reference: Black Screen on Power-On (Task 1)

# 1. Problem Statement

A desktop computer does not start properly. After the power button is pressed, the system fails to display any output on the monitor, resulting in a black screen. The objective of this documentation is to identify the most probable causes of the fault and to provide a structured, repeatable diagnostic and repair procedure that a technician can follow to resolve the issue.

# 2. Symptom Summary

- Computer is powered on (button pressed) but no image appears on the monitor.

- Screen remains completely black throughout the startup attempt.

- Fans, lights, or beeps may or may not be present, depending on the underlying cause.

# 3. Possible Causes

Before beginning a physical inspection, it is useful to review the most likely causes of a black-screen startup failure. The table below ranks common causes by likelihood and lists the symptom usually associated with each.

| Possible Cause | Likelihood | Typical Symptom |
| --- | --- | --- |
| Loose or disconnected power cable | High | No fans/lights at all, or intermittent power |
| Loose RAM module(s) | High | Series of beeps, no display, system powers on |
| Loose or faulty video cable / GPU | High | Fans spin, no display on monitor |
| Faulty or wrong display input selected | Medium | Monitor shows “No Signal” |
| Faulty Power Supply Unit (PSU) | Medium | No power at all, burning smell, clicking sounds |
| Corrupted BIOS/UEFI or CMOS battery failure | Medium | System boots then halts, repeated beep codes |
| Faulty motherboard | Low | No POST, no beeps, no fan response |
| Overheating CPU/GPU triggering shutdown | Low | Brief power-up, then immediate shutdown |

# 4. Tools and Materials Required

| Tool/Item | Purpose |
| --- | --- |
| Phillips screwdriver | Open the computer case |
| Anti-static wrist strap | Prevent electrostatic discharge damage to components |
| Spare known-good power cable | Test for faulty cable |
| Spare known-good monitor/video cable | Test for faulty cable or port |
| Multimeter (optional) | Test PSU output voltages |
| Compressed air | Clean dust from components |

# 5. Safety Precautions

- Power off the system and unplug it from the wall outlet before opening the case.

- Wear an anti-static wrist strap or regularly touch a grounded metal surface to avoid electrostatic discharge (ESD).

- Handle all internal components by their edges; avoid touching gold contacts or chips directly.

- Do not force any cable or component connection.

# 6. Diagnostic and Repair Procedure

## Step 1 – Check External Connections

- Confirm the monitor is powered on and the correct input source (HDMI/DisplayPort/VGA) is selected.

- Check that the video cable is firmly connected at both the monitor and the computer ports.

- Verify the power cable is securely connected to the PSU and the wall outlet, and that the outlet is supplying power (test with another device).

- If using a surge protector or power strip, confirm it is switched on.

## Step 2 – Observe Power-On Behavior

- Press the power button and observe whether fans spin, lights illuminate, or beep codes sound.

- No response at all (no fans, no lights) indicates a power-related problem; proceed to Step 3.

- Fans/lights active but no display indicates a video or memory-related problem; proceed to Step 4.

## Step 3 – Investigate Power Issues

- Disconnect the power cable, open the case following safety precautions in Section 5.

- Inspect the 24-pin motherboard connector and 4/8-pin CPU power connector for secure seating.

- Test the wall outlet and power cable with another known-good device or cable.

- If available, test the PSU output with a multimeter or a PSU tester.

- If the PSU is confirmed faulty, replace it and retest.

## Step 4 – Investigate Video and Memory Issues

- With the system powered off and unplugged, reseat the RAM modules by removing and firmly reinserting them in their slots.

- If using a dedicated graphics card, reseat it in the PCIe slot and confirm any auxiliary power connectors are attached.

- Test the monitor and video cable on another computer to rule out a display fault.

- If the motherboard has integrated graphics, connect the monitor directly to the motherboard video output (after removing the dedicated GPU) to isolate the fault to the GPU.

- Power on the system and listen for beep codes, which can indicate a specific memory or video fault (refer to the motherboard manufacturer’s beep code chart).

## Step 5 – Check for Overheating or Component Failure

- Inspect CPU and case fans for proper operation and excessive dust buildup; clean with compressed air if needed.

- Confirm the CPU heatsink is properly seated and making good contact.

- If the system powers on briefly and then shuts down, this may indicate a thermal protection shutdown or a failing component.

## Step 6 – Reset BIOS/CMOS

- Locate the CMOS battery on the motherboard and remove it for 1–5 minutes, then reinsert it (or use the CMOS reset jumper/button if available).

- This clears any corrupted BIOS settings that may be preventing a successful boot.

- Power on the system and check whether the display returns.

## Step 7 – Isolate the Faulty Component

- If the issue persists, disconnect all non-essential components (extra drives, peripherals, expansion cards) and boot with a minimal configuration.

- Reintroduce components one at a time, testing after each addition, to identify the component causing the fault.

- If no combination resolves the issue, the motherboard or CPU may be faulty and should be tested in isolation or replaced.

# 7. Verification

Once a corrective action has been taken, restart the computer and confirm that the system completes POST (Power-On Self-Test) and displays the manufacturer logo or operating system boot screen. Allow the system to fully boot into the operating system and run for several minutes to confirm stability before considering the repair complete.

# 8. Documentation and Reporting

- Record the date, time, and reported symptoms at the start of the repair.

- Document each diagnostic step performed and its outcome.

- Record the root cause identified and the corrective action taken.

- Note any replacement parts used, including make/model and serial numbers where applicable.

- Confirm and record successful verification of the repair.

# 9. Conclusion

A black screen on startup is most commonly caused by a loose connection, faulty cable, or improperly seated component rather than a major hardware failure. Following a structured, step-by-step elimination process — starting with the simplest and least costly checks — allows the technician to efficiently identify and resolve the root cause while minimizing unnecessary part replacement.